| Tender texts as templates for construction projects in which a tender will be issued Product: CORTIFLEX160 Straight Shape |  |
| --- | --- |

Path and flower bed laser cut metal edging made of Corten steel, Cortiflex® profile,
straight shape, deliver and install in accordance with the manufacturer's installation instructions.
Flexible profile with square holes for screwing in concrete and ground anchors and other accessories.

Item:		Corten steel rail with 6 laser-cut anchor connection screw holes
Item no.:	C160N24
Profile height:	160 mm
Length:	2,400 mm 
Thickness:	3 mm 
Colour:	Steel grey, rusting

Item:		Concrete and ground anchors with 6 laser-cut anchor / connection screw holes
		for fastening the 160 mm profiles
Item no.:	CBA15
Width:		30-120 mm
Length:	300 mm
Thickness:	3 mm
Colour:	Steel grey, rusting

Item:		Concrete and ground anchor with 2 laser-cut anchor / connection screw holes
		for fastening the 105, 160, 200 and 300 mm profiles
Item no.:	CBA10
Width:		30 mm
Length:	200 mm
Thickness:	3 mm
Colour:	Steel grey, rusting

Item:		M8 screw, washer + screw nut
Item no.:	SV2AM8100
Length:	approx. 20 mm 
Material:	V2A stainless steel

Manufacturer:QualityEdging FZ LLC
	Tel./WhatsApp: +971 50 3464005
	www.qualityedging.com
	E-Mail: sales@qualityedging.com
	Compass Building, Al Shohada Road
	Al Hamra Industrial Zone
	Ras Al Khaimah, United Arab Emirates